Microprocessor Systems: Principles and Implementation, Fall 2024 Midterm Online Examination

Goal

In this online exam, you will modify the Aquila processor to determine the maximal runtime stack usage of a program. A prebuilt binary of coremark.elf and coremark.objdump can be downloaded from E3 as a test program. The TA will use a different program to test your code after the exam so you shall not hardcode the solution into your code. You can use the behavioral simulation of Vivado to simulate the execution of it. You have to modify the testbench module aquila_tb.v such that when the simulation is finished, it will display the maximal runtime stack size used by the program in words.

| Exam Regulations |  |
| --- | --- |
| [1] | You are not allowed to use your mobile phone, browse the internet for any references, and talk to any other classmates. If you have any questions, please raise your hand and the TA will talk to you. |
| [2] | You are allowed 2 hours for this online exam. (10:10 AM to 12:00 PM) |
| [3] | You can download your previous homework code and the Aquila workspace, or the Power Point slides from E3 to the test computer during exam. |
| [4] | After your circuit is completed and verified, please upload all your modified Verilog files to E3. Do not upload the entire Vivado workspace. You have to leave the computer classroom immediately afterwards. |

Exam Descriptions

For this exam, you must modify the circuit of Aquila (use the aquila_build.zip provided in HW0) to compute the maximal runtime stack size after the execution of an ELF program. Note that you do not add new files/modules to Aquila. Just modify the existing code and upload the modified Verilog files.

Note that you only count the stack usage of the ELF program. Do not count the stack usage of the uartboot loader. That is, you should record the stack pointer right before entering the main function and monitor the growth and shrink of the stack pointer to determine the maximal stack usage. When simulation ends, display the maximal stack size used by the ELF program at the TCL console in aquila_tb.v.

wire [31 : 0] max_stack_words;

always @(posedge clk)

begin

if (simulation_finished) begin

$display();

$display("Maximal runtime stack size: ", max_stack_words);

$display();

$display("Simulation finished.");

$finish();

end

end

Figure 1. Code segment that prints cycles before the simulation finished.